# ReadMe File for FAST v8.09.00a-bjj

Bonnie Jonkman, Jason Jonkman
National Renewable Energy Laboratory
September 30, 2014

# Table of Contents

Table of Contents	1

Introduction	2

Major changes in FAST	7

v8.09.00a-bjj	7

v8.08.00c-bjj	8

v8.03.02b-bjj	12

Certification Tests	12

Variables Specified in the FAST Primary Input File	13

Simulation Control	13

Feature Switches and Flags	15

Input Files	16

Output	17

Modeling Tips	19

Limitations	19

Future Work	19

Converting to FAST v8.09.x	20

Summary of Changes to Inputs	20

Matlab Conversion Scripts	22

Compiling	23

Running FAST v8.09.00a-bjj	24

Feedback	24

Appendix: Example FAST v8.09.* Input File	25

# Introduction

This document is designed to guide you through some of the changes that the FAST wind turbine computer-aided engineering (CAE) tool is undergoing. FAST v8.09.00c-bjj is the third public release of FAST under the new modularization framework developed at NREL. The architecture of FAST v8 is entirely different from FAST v7.02.00d-bjj. These differences are highlighted in Figure 1.

The modules of FAST (AeroDyn, HydroDyn, etc.) correspond to different physical domains of the coupled aero-hydro-servo-elastic solution, most of which are separated by spatial boundaries. Figure 2 shows the control volumes associated with each module for fixed-bottom offshore wind turbines. For land-based wind turbines, the HydroDyn hydrodynamics module would not be used and the SubDyn multi-member substructure structural-dynamics module is optional. Figure 3 shows the control volumes associated with each module for floating offshore wind turbines.

While FAST v8 has many features not found in FAST v7, several features of FAST v7.02.00d-bjj have not yet been added to FAST v8, so we will continue to support both versions of the software (FAST v7 and FAST v8) until FAST v8 is deemed a suitable replacement. Table 1 summarizes the different features available in each version.

Figure 1: Architectural comparison of FAST 7 and FAST 8

Figure 2: FAST control volumes for fixed-bottom systems (surface ice not shown)

Figure 3: FAST control volumes for floating systems

Table 1: Comparison of features between FAST v7 and v8

| Aerodynamics (AeroDyn and InflowWind) |  |  |
| --- | --- | --- |
| FAST Features | v7.02 | v8.09 |
| • Quasi-steady or dynamic wake |  |  |
| • Steady or unsteady airfoil aerodynamics |  |  |
| • Tower shadow for downwind rotors |  |  |
| • Tower influence for upwind rotors |  |  |
| • Tower drag loading |  |  |
| • Tail-fin aerodynamic loading |  |  |
| • "Hub-height", TurbSim, and GH Bladed wind file formats |  |  |
| • Other wind formats |  |  |
| • Aeroacoustics (noise) |  |  |
| Hydrodynamics (HydroDyn, IceFloe, and IceDyn) |  |  |
| FAST Features | v7.02 | v8.09 |
| • First-order regular or irregular waves |  |  |
| • Second-order difference- and sum-frequency wave terms |  |  |
| • White-noise waves |  |  |
| • Wave directional spreading |  |  |
| • Wave stretching |  |  |
| • Externally generated wave data |  |  |
| • Sea current |  |  |
| • Strip theory for central member |  |  |
| • Strip theory for multiple intersecting members |  |  |
| • Distributed static buoyancy |  |  |
| • Concentrated loads on member ends |  |  |
| • Support for inclined and tapered members |  |  |
| • Support for flooded and ballasted members |  |  |
| • Support for marine growth |  |  |
| • First-order potential flow (via WAMIT) |  |  |
| • Second-order diff.- and sum-frequency potential-flow terms (via WAMIT) |  |  |
| • Radiation "memory effect" captured through time-domain convolution |  |  |
| • Radiation "memory effect" captured through linear state-space form |  |  |
| • Quasi-steady and dynamic surface-ice loading |  |  |

| Control and Electrical System (Servo) Dynamics (ServoDyn) |  |  |
| --- | --- | --- |
| FAST Features | v7.02 | v8.09 |
| • Blade-pitch control |  |  |
| • Override pitch maneuvers |  |  |
| • Generator models |  |  |
| • Torque control |  |  |
| • High-speed shaft brake |  |  |
| • Nacelle-yaw control |  |  |
| • Override yaw maneuvers |  |  |
| • Blade-tip brakes |  |  |
| • GH Bladed DLL interface |  |  |
| • Simulink interface |  |  |
| • LabVIEW interface |  |  |
| Structural Dynamics (ElastoDyn, SubDyn, MAP, and FEAMooring) |  |  |
| FAST Features | v7.02 | v8.09 |
| • Blade-bending degrees-of-freedom (DOFs) |  |  |
| • Rotor-teeter DOF |  |  |
| • Generator azimuth and drivetrain torsion DOFs |  |  |
| • Nacelle-yaw DOF |  |  |
| • Tower-bending DOFs |  |  |
| • Rigid-body platform DOFs |  |  |
| • Furling DOFs |  |  |
| • Fixed-bottom multi-member substructure DOFs |  |  |
| • Gravitational loading |  |  |
| • Gearbox friction |  |  |
| • System of independent mooring lines solved quasi-statically |  |  |
| • System of multi-segmented mooring lines solved quasi-statically |  |  |
| • Mooring dynamics |  |  |
| • Earthquake excitation |  |  |

| General |  |  |
| --- | --- | --- |
| FAST Features | v7.02 | v8.09 |
| • Time marching |  |  |
| • Operating-point determination |  |  |
| • Linearization |  |  |
| • FAST-to-ADAMS preprocessor |  |  |
| • Follows the new FAST modularization framework |  |  |
| • Structural and control routines separated from driver code |  |  |
| • Independent time steps between modules |  |  |
| • Independent spatial discretization between modules |  |  |
| • Multiple integration options |  |  |
| • Loose coupling with predictor-corrector across modules |  |  |
| • Both 32-bit and 64-bit applications available |  |  |
| • Supports both Windows and Linux operating systems |  |  |
| • Optimized for efficiency |  |  |
| • Supports mixed Fortran/C |  |  |
| • Compiles with gfortran |  |  |

# Major changes in FAST

## v8.10.00a-bjj

## MAP++

## TMD (tuned mass damper)

## MoorDyn

## FASTMooring

## IceDyn

## IceFloe (official?)

## FAST v8 Interface for Simulink

## HydroDyn bug fixes

## v8.09.00a-bjj

- We added second-order wave kinematics and second-order diffraction loading to HydroDyn.  Before this update, HydroDyn was previously based solely on first-order hydrodynamics theory.  The second-order hydrodynamic implementations include time-domain calculations of difference- (mean- and slow-drift-) and sum-frequency terms.  Second-order wave kinematics are applicable to the hydrodynamic loading of thin structural members via strip theory and second-order diffraction loads are applicable to large structural members.

- We fixed a problem where the eigenfrequencies listed in the SubDyn summary file were incorrect for some models.

- We fixed a problem with the tower distributed with the OC3-tripod model in the FAST certification test (Test20).

- We added source code for the DWM sub-module in AeroDyn. This feature was developed and will be supported by researchers at the University of Massachusetts; if you wish to use the DWM feature, you must obtain the DWM driver code, distributed separately: https://nwtc.nrel.gov/DWM.

FAST v8.09.00a-bjj is compiled with the components listed in Table 2.

Table 2: Components in FAST v8.09.00a-bjj

| Component | Version |  |
| --- | --- | --- |
| Modules |  | ModName (for output files) |
| ElastoDyn | v1.01.07a-bjj | ED |
| AeroDyn | v14.03.00a-bjj | AD |
| InflowWind | v2.00.01c-bjj | IfW |
| ServoDyn | v1.01.02a-bjj | SrvD |
| HydroDyn | v2.02.00a-adp | HD |
| SubDyn | v1.01.01a-rrd | SD |
| MAP | v0.97.01a-mdm | MAP |
| FEAMooring | v1.00.00-yhb | FEAM |
| IceFloe | v1.00.00 | IceF |
| IceDyn | v1.00.01-by | IceD |
| Other Components |  |  |
| NWTC Subroutine Library | v2.04.00a-bjj |  |
| FAST Registry | v2.03.02 |  |
| Third Party Content |  |  |
| LAPACK | v3.3 as part of Intel® Math Kernel Library; (v3.5.0 compiled with gfortran) |  |
| ScaLAPACK | 2.0.2 |  |
| FFTPACK | v4.1 |  |

## v8.08.00c-bjj

- Coupling between ElastoDyn, SubDyn, and HydroDyn was added, allowing FAST to model fixed-bottom offshore turbines, including multi-member substructures (e.g., tripods and jackets).

- The module input-output solves have been enhanced; see the following paper for theoretical details: http://www.nrel.gov/docs/fy14osti/60742.pdf.

- The mesh mapping algorithms have been enhanced; see the following paper for theoretical details: http://www.nrel.gov/docs/fy14osti/60742.pdf.

- We now use LAPACK routines for solving linear systems, which has increased the speed of the simulations.

- The glue code allows the option for time-step subcycling. Modules can now choose to use smaller time steps than the glue code, as long as the module time step is an integer divisor of the glue-code time step. Note that we have found no cases where this option would be necessary.

- New modules for ice loading were added: IceFloe and IceDyn.

- Another module for mooring lines was added: FEAMooring‡‡.

- The names of output files generated by both FAST and its modules have been standardized, see Figure 4. Files generated by FAST are named
			<RootName>.<ext>
and files generated by FAST modules are named 
			<RootName>.<ModName>.<ext> 
where <RootName> is the root name of the primary FAST input file (the file name, including path, without the extension), <ModName> is an abbreviation for the module generating the file, and <ext> is the file extension. File extensions currently are

| Output file extension | File type |
| --- | --- |
| sum | Summary file |
| out | Time-marching tabular text output |
| outb | Time-marching tabular binary output |
| ech | Echo of input file (primarily for debugging) |

- The “Time Ratio” displayed at the end of a simulation now includes only the CPU time after initialization. This ratio was changed to help users better predict the amount of time longer simulations will take (e.g., extrapolating the time a 1-hr simulation will take based on running a 1-min simulation).

- Information about the Jacobian and time steps was added to the FAST summary file.

- Bugs in handling errors were fixed. (In FAST v8.03.02b-bjj, InflowWind did not return all of its errors to the glue code, so it was using zero wind velocity when it went outside the turbulence grid.)

- FAST no longer allows the tower influence model, “NEWTOWER,” to be used in AeroDyn on floating offshore turbines. This tower influence model assumes the tower does not move, which is a poor assumption for floating turbines.

- FAST will now abort if ElastoDyn’s TowerBsHt value is negative for floating offshore systems.

- We fixed a bug in ElastoDyn (which is also present in FAST v7.02.00d) where the linear teeter-damper moment did not use TeetDmpP.

- We fixed a problem where the ElastoDyn Azimuth channel would be negative in rare cases.

- We fixed a problem with ElastoDyn’s implementation of AM4, which incorrectly initialized the state history if corrections steps were taken.

- We no longer allow extrapolation orders of 0 in FAST v8.08.00c-bjj.

- We updated the DISCON*.DLL files used in the 5MW model certification tests. Previously, they did not work if users did not have Intel Visual Fortran installed on the computers they ran the simulations on.

- We fixed some bugs in the AeroDyn input files of the NREL 5-MW land-based turbine.

- We set the time steps of the floating offshore certification tests to be the same as they were in FAST v7.02.00d

- We added certification tests for the OC3 Monopile, OC3 Tripod, OC4 Jacket, and OC4-DeepCwind Semi-Submersible models.

- We have added some more utility files to the FAST archive, including:

  - PlotFASToutput.m, a MATLAB function that can plot some or all channels of one or more FAST time-series output files.

  - ReadSubDynSummary.m, a MATLAB function that can read the SubDyn summary file and put the data into a Matlab data-structure.

- We have updated the MAP_win32.dll file distributed with FAST so that it no longer depends on python being installed on the computer running FAST.

- We have added a 64-bit FAST executable to the archive, as well as a 64-bit version of DISCON_win64.DLL, and a “dummy” 64-bit version of MAP. This executable may be useful for running long simulations of large fixed-bottom offshore models (e.g., the OC4 Jacket); it cannot run any models that want to call the MAP module.

## v8.10.00a-bjj

- The 64-bit version of the MAP DLL is now a functioning. This means all functionality in the FAST_Win32.exe is available in FAST_x64.exe

Figure 4: Summary of Input and Output Files for FAST v8.09.00a-bjj

## v8.03.02b-bjj

Tasks completed to develop FAST v8 included:

- Converted FAST and its various modules (including AeroDyn and HydroDyn) into the new modularization framework (splitting out the controls and electrical-drive dynamics into a new ServoDyn module and structural dynamics into a new ElastoDyn module),

- Implemented a new driver program (glue code) supporting loose coupling of the modules,

- Developed mesh-to-mesh mapping schemes between module-independent discretizations of the spatial boundaries between modules,

- Coupled in the recently developed SubDyn module for multi-member substructure structural dynamics and MAP module for multi-segmented mooring quasi-statics, and

- Included a series of models using the NREL 5-MW Baseline wind turbine in the CertTest, including offshore configurations.

The driver program (glue code) couples the modules together; it controls the overall simulation progress and maps module outputs to inputs. We use the name “FAST” both for the driver program (glue code) and overall coupled code.

FAST and each of its modules, except InflowWind, have their own input files; see Figure 4.

# Certification Tests

Several new models were added to the FAST v8.08.00c-bjj archive. Table 3 lists the tests (1-25) and models available in the FAST CertTest folder:

Table 3: Certification Tests Distributed with FAST v8.09.00a-bjj

| Test Name | Turbine Name | No. Blades (-) | Rotor Diameter (m) | Rated Power (kW) | Test Description |
| --- | --- | --- | --- | --- | --- |
| Test01 | AWT-27CR2 | 2 | 27 | 175 | Flexible, fixed yaw error, steady wind |
| Test02 | AWT-27CR2 | 2 | 27 | 175 | Flexible, steady wind |
| Test03 | AWT-27CR2 | 2 | 27 | 175 | Flexible, free yaw, steady wind |
| Test04 | AWT-27CR2 | 2 | 27 | 175 | Flexible, free yaw, turbulence |
| Test05 | AWT-27CR2 | 2 | 27 | 175 | Flexible, steady wind |
| Test06 | AOC-15/50 | 3 | 15 | 50 | Flexible, steady wind |
| Test07 | AOC-15/50 | 3 | 15 | 50 | Flexible, free yaw, turbulence |
| Test08 | AOC-15/50 | 3 | 15 | 50 | Flexible, fixed yaw error, steady wind |
| Test09 | UAE VI downwind | 2 | 10 | 20 | Flexible, yaw ramp, steady wind |
| Test10 | UAE VI upwind | 2 | 10 | 20 | Rigid, power curve, ramp wind |
| Test11 | WP 1.5 MW | 3 | 70 | 1500 | Flexible, variable speed & pitch control, pitch failure, turbulence |
| Test12 | WP 1.5 MW | 3 | 70 | 1500 | Flexible, variable speed & pitch control, ECD event |
| Test13 | WP 1.5 MW | 3 | 70 | 1500 | Flexible, variable speed & pitch control, turbulence |
| Test14 | Not currently available |  |  |  |  |
| Test15 | SWRT | 3 | 5.8 | 10 | Flexible, variable speed control, free yaw, EOG01 event |
| Test16 | SWRT | 3 | 5.8 | 10 | Flexible, variable speed control, free yaw, EDC01 event |
| Test17 | SWRT | 3 | 5.8 | 10 | Flexible, variable speed control, free yaw, turbulence |
| Test18 | NREL 5 MW - Land-based | 3 | 126 | 5000 | Flexible, DLL control, tower potential flow and drag, turbulence |
| Test19 | NREL 5 MW - OC3-Monopile | 3 | 126 | 5000 | Flexible, DLL control, tower potential flow, turbulence, irregular waves |
| Test20 | NREL 5 MW - OC3-Tripod | 3 | 126 | 5000 | Flexible, DLL control, tower potential flow, steady wind, regular waves with 0 phase |
| Test21 | NREL 5 MW - OC4-Jacket | 3 | 126 | 5000 | Flexible, DLL control, tower potential flow, turbulence, irregular waves, marine growth |
| Test22 | NREL 5 MW - ITI Barge | 3 | 126 | 5000 | Flexible, DLL control, irregular & multidirectional waves, turbulence |
| Test23 | NREL 5 MW - MIT/NREL TLP | 3 | 126 | 5000 | Flexible, DLL control, turbulence, irregular waves |
| Test24 | NREL 5 MW - OC3-Hywind | 3 | 126 | 5000 | Flexible, DLL control, turbulence, irregular waves |
| Test25 | NREL 5 MW - OC4-DeepCwind Semi-Submersible | 3 | 126 | 5000 | Shortened OC4 Load Case (LC) 3.7: steady wind, white noise waves |

# Variables Specified in the FAST Primary Input File

FAST expects to find variables on specific lines in the text input file, so, do not add or remove lines in the file. The inputs listed in the file are described below, and an example file is provided at the end of this document, in Appendix: Example FAST v8.09.* Input File.

## Simulation Control

This section of the input file contains options for controlling the simulation.

### Echo: Echo input data to <RootName>.ech [T/F]

Setting this flag to “True” will result in the FAST primary input file being echoed to a file named “<RootName>.ech” where <RootName> is the name of the FAST primary input file, excluding its file extension. This feature is useful for debugging an input file.

### AbortLevel: Error level when simulation should abort [“WARNING”, “SEVERE”, or “FATAL”]

This string tells FAST what error level should cause an abort. Typically we set this to abort on fatal errors, but there may be instances when a user wishes to abort on severe errors or warnings.

Fatal errors are those from which the program cannot recover. For example:

- Running out of memory when trying to allocate space for variables.

- Trying to read a number from a line of an input file that does not contain numeric values.

- Reaching the end of an input file before reading all the necessary information.

- Trying to open a file for writing, but the file is locked from another process.

Some examples of severe errors include the following:

- A format specifier for real numbers is too narrow to print “–1.0”, so output files will almost certainly contain “***” instead of actual numbers.

- When trying to read a numeric value, logical “True” or “False” values were found instead. Fortran interprets them as 0 or 1, but that may not be what the user intended.

- A routine is using math based on the assumption that the angles are small, but the angles it found were larger than what it considers “small.”

Warnings are typically generated when the simulation can continue—perhaps by the program adjusting inputs—but the results may not be what the user expected. Things that may generate warnings include

- Cases when user inputs are modified:

  - If the user asked for output on more tower strain gages than there are tower nodes, ElastoDyn will set the number of strain gages equal to the number of nodes.

  - If air density is set to zero, AeroDyn will turn off the dynamic-inflow model.

- Cases where non-physical conditions could be modeled:

  - If the user enables ElastoDyn’s second flap mode but does not enable the first flap mode.

  - If the user has disabled wake calculations in AeroDyn.

### TMax: Total run time [s]

This is the total length of the simulation to be run, in seconds. The first output is calculated at t = 0; the last output is calculated at t = TMax seconds.

### DT: Recommended module time step [s]

This is the global, or glue-code, time step, DT is the value FAST will suggest modules use, although some modules may choose to use a time step that is an integer multiple smaller than DT. Module input-output relationships used to couple the modules together are calculated every DT seconds.  It is essential that a small enough time step is used to ensure solution accuracy (by providing a sufficient sampling rate to characterize all key frequencies of the system), to ensure numerical stability of the selected time-integrators, and to ensure that the coupling between modules of FAST is numerically stable.

Our rule of thumb is to set DT = 1 / (10 * highest natural frequency in Hz of coupling between modules). This natural frequency is hard to estimate before the full-system linearization of the coupled FAST model is realized. For coupled FAST models that don’t use SubDyn, the frequency can be estimated via a linearization analysis of FAST v7.  For coupled FAST models that do use SubDyn, guidance for choosing the time step is found in the SubDyn ReadMe file.

### InterpOrder: Interpolation/Extrapolation order for input/output time history [1 or 2]

This is the order of the interpolation or extrapolation used for module inputs in the FAST glue code. Valid entries are “1” for linear interpolation/extrapolation or “2” for quadratic interpolation/extrapolation. Previous module inputs are extrapolated at the beginning of each step in the time-advancement loop to provide a guess for the actual module inputs at that step. Module inputs are typically interpolated in a module’s UpdateStates routine.

We have found that quadratic extrapolation typically works well. However, there are times when linear extrapolation provides a stable solution while quadratic does not. We have found this to be true for cases where the model has poor initial values or cases where the simulation may have errors building up.

### NumCrctn: Number of correction iterations [-]

This is the number of corrections to be taken on each step of the predictor-corrector scheme implemented in FAST. The value of NumCrctn must not be negative. Most models can achieve stable solutions by using explicit calculations (i.e., no corrections: NumCrctn = 0), particularly if using InterpOrder = 2 and the recommended DT—see above. However, corrections may be needed if you wish to achieve a given convergence rate of an underlying time integrator (e.g., if you are using a 4th-order accurate integration scheme, you may only get a 2nd-order accurate solution with no corrections. If you desire a 4th-order accurate solution, you may need one or more corrections).

### DT_UJac: Time between calls to get Jacobians [s]

We use a Jacobian matrix to solve the module input-output relationship between accelerations and loads in the ElastoDyn-HydroDyn-SubDyn coupling. This Jacobian is computed with finite differences and can be time consuming. However, it rarely needs to be calculated frequently.

DT_UJac determines how often the Jacobian needs to be updated. If the platform reference point in ElastoDyn doesn't rotate much, DT_UJac can be set to a value larger than TMax. DT_UJac is not currently used for land-based systems. For floating systems, where the platform may rotate more than several degrees in roll, pitch, and/or yaw, it is recommend to set DT_UJac = 1/(10*natural frequency in Hz of the roll, pitch, or yaw mode with excessive motion).

### UJacSclFact: Scaling factor used in Jacobians [-]

This factor is used to divide the magnitude of the load terms in the Jacobian (see DT_UJac) so that they are approximately the same order of magnitude as the acceleration terms. For the NREL 5-MW turbine models in the Certification Test, we’ve set it to 1E+06 and have not found any cases where that value did not work. UJacSclFact may need to be set larger or smaller when modeling wind turbines much larger or smaller turbines than the NREL 5-MW baseline.

## Feature Switches and Flags

This section of the input file contains switches and flags that tell FAST which modules should be used in the simulation.

### CompElast: Compute structural dynamics [1 or 2]

1: Use ElastoDyn for the structural dynamics of the rotor, drivetrain, nacelle, and tower
2: Use BeamDyn for the structural dynamics on the blades and ElastoDyn for the drivetrain, nacelle, and tower. CompElast = 2 is currently disabled; it will be implemented when BeamDyn is integrated into FAST.

Please note that ElastoDyn must always be used when running FAST.

### CompAero: Compute aerodynamic loads [0 or 1]

0: Do not calculate aerodynamic loads
1: Use AeroDyn for aerodynamic loads

### CompServo: Compute control and electrical-drive dynamics [0 or 1]

0: Do not calculate control and electrical-drive dynamics
1: Use ServoDyn for control and electrical-drive dynamics

### CompHydro: Compute hydrodynamic loads [0 or 1]

0: Do not calculate hydrodynamic loads
1: Use HydroDyn for hydrodynamic loads

If CompHydro is not zero, FAST considers the model to be an offshore system. If CompSub is also non-zero, the offshore system is a fixed-bottom system. If CompSub is zero, the offshore system is considered a floating system.

### CompSub: Compute sub-structural dynamics [0 or 1]

0: Do not calculate sub-structural dynamics
1: Use SubDyn for sub-structural dynamics

### CompMooring: Compute mooring system [0, 1, or 2]

0: Do not model a mooring system
1: Use MAP to model a mooring system
2: Use FEAMooring to model a mooring system

Note that FEAMooring is not complete (not fully functional) in FAST v8.09.00a-bjj.

### CompIce: Compute ice loads [0, 1, or 2]

0: Do not model offshore surface ice
1: Use IceFloe to model offshore surface ice
2: Use IceDyn to model offshore surface ice

Note that IceDyn is not complete (not fully functional) in FAST v8.09.00a-bjj.

### CompUserPtfmLd: Compute additional platform loading [T/F]

This feature is currently disabled.

### CompUserTwrLd: Compute additional tower loading [T/F]

This feature is currently disabled.

## Input Files

The input files specified in this section can be specified relative to the location of the FAST primary input file or specified with an absolute path. We recommend that you use quotes around the path/file names.

### EDFile: Name of file containing ElastoDyn input parameters [-]

This is the name of the ElastoDyn primary input file.

### BDBldFile(1): Name of file containing BeamDyn input parameters for blade 1 [-]

This feature is currently disabled.

### BDBldFile(2): Name of file containing BeamDyn input parameters for blade 2 [-]

This feature is currently disabled.

### BDBldFile(3): Name of file containing BeamDyn input parameters for blade 3 [-]

This feature is currently disabled.

### AeroFile: Name of file containing aerodynamic input parameters [-]

This is the name of the AeroDyn primary input file. At this time, AeroFile must be specified even if CompAero = 0 because ElastoDyn currently reads the AeroDyn input file for its blade discretization. We plan to fix this to use the mesh-mapping required in the FAST framework in a future release.

### ServoFile: Name of file containing control and electrical-drive input parameters [-]

This is the name of the ServoDyn primary input file. It is not used if CompServo = 0.

### HydroFile: Name of file containing hydrodynamic input parameters [-]

This is the name of the HydroDyn primary input file. It is not used if CompHydro = 0.

### SubFile: Name of file containing sub-structural input parameters [-]

This is the name of the SubDyn primary input file. It is not used if CompSub = 0.

### MooringFile: Name of file containing mooring system input parameters [-]

This is the name of the MAP (CompMooring = 1) or FEAMooring (CompMooring = 2) primary input file. It is not used if CompMooring = 0.

### IceFile: Name of file containing ice input parameters [-]

This is the name of the IceFloe (CompIce = 1) or IceDyn (CompIce = 2) primary input file. It is not used if CompIce = 0.

## Output

This section deals with what can be output from a FAST simulation.

### SumPrint: Print summary data to “<RootName>.sum” [T/F]

When set to “true”, FAST will generate a file named “<RootName>.sum”. This summary file contains the version number of all modules being used, the time steps for each module, and information about the channels being written to the time-marching output file(s). If SumPrint is “false”, no summary file will be generated.

### SttsTime: Amount of time between screen status messages [s]

During a FAST simulation, the program prints a message like this:

SttsTime sets how frequently this message is updated. For example, if SttsTime is 2 seconds, you will see this message updated every 2 seconds of simulation time.

### DT_Out: Time step for tabular output [s]

This is the time step of the data in the tabular (time-marching) output files. DT_Out must be an integer multiple of DT.

### TStart: Time to begin tabular output [s]

This is time step that must be reached before FAST will begin writing data in the tabular (time-marching) output files. Note that the output files may not actually start at TStart seconds if TStart is not an integer multiple of DT_Out.

### OutFileFmt: Format for tabular output [1, 2, or 3]

This indicates which type of tabular (time-marching) output files will be generated. If OutFileFmt is 1, only a text file will be written. If OutFileFmt is 2, only a binary file will be written. If OutFileFmt is 3, both text and binary files will be written.

Text files write a line to the file each time step. This can make the simulation run slower, but it can be useful for debugging, particularly if a simulation doesn’t run to completion or if you want to look at some results before the entire simulation finishes.

Binary files write the entire file at the end of the simulation. If a lot of output channels are requested for a long simulation, this can take up a moderate amount of memory. However, they tend to run faster and the resulting files take up much less space. The binary files contain more precise output data than text files, which are limited by the chosen output format specifier—see OutFmt below.

We recommend you use text files for debugging and binary files for production work. A MATLAB script for reading FAST binary output files is included in the archive (see Utilities\SimulationToolbox\Utilities\ReadFASTbinary.m). The NREL post-processors Crunch and MCrunch can also read these binary files.

### TabDelim: Use tab delimiters in text tabular output file? [T/F]

When OutFileFmt = 1 or 3, setting TabDelim to “True” will put tabs between columns in the text tabular output file. Otherwise, spaces will separate columns in the text tabular output file. If OutFileFmt = 2, TabDelim has no effect.

### OutFmt: Format used for text tabular output, excluding the time channel [-]

When OutFileFmt = 1 or 3, FAST will use OutFmt to format the channels printed in the text tabular output file. OutFmt should result in a field that is 10 characters long (channel headers are 10 characters long, and NWTC post-processing software sometimes assume 10 characters). The time channel is printed using the “F10.4” format. We commonly specify OutFmt to be “ES10.3E2”.

If OutFileFmt = 2, OutFmt has no effect.

# Modeling Tips

If a model is numerically unstable, you can try these steps

- Add a correction step (NumCrctn).

- Make DT smaller.

- Change InterpOrder.

- Set better initial conditions in the module input files (particularly ElastoDyn).

- Simplify models (e.g. disable modules and eliminate DOFs) to debug problems

# Limitations

Table 1 shows a comparison of features between FAST v7 and FAST v8.09.00c-bjj. It is our intent to include all of the features of FAST v7 in the new modular framework of FAST v8 in the future. Until then, NREL plans to support both versions of FAST. Other limitations of FAST v8 include:

- FAST v8.09.00c-bjj is distributed as both a 32-bit and 64-bit Windows application. However, the 64-bit application cannot run the MAP module. We are working to update the MAP DLL so that it does not depend on third-party libraries that are not supported in 64-bit Windows applications.

- FAST v8.09. 00c-bjj runs a bit slower than FAST v7.02.00d-bjj (though the offshore cases run significantly faster than FAST v8.03.02b-bjj). We have put our effort into getting the framework to work and will address computational efficiency later. We expect great improvements in efficiency as development continues.

- The OC4 Jacket model requires approximately 2 GB of memory and may not run on 32-bit Windows® systems. (The model does run using FAST_Win32.exe on a 64-bit Windows® system.)

- The IceDyn and FEAMooring modules are not complete in FAST v8.09.00c-bjj.

# Future Work

All future developments of FAST will follow the framework.

- Address items from the “Limitations” section

- Introduce the new BeamDyn module for nonlinear finite-element modeling of blade structural dynamics

- Upgrade the loose-coupling algorithm of the glue code to allow each module to have its own time step, including time steps larger than the glue-code time step.

- Upgrade ElastoDyn and AeroDyn to have distinct discretization schemes for the blades and tower (currently ElastoDyn uses AeroDyn’s blade discretization and AeroDyn uses ElastoDyn’s tower discretization)

- Optimize the code, particularly ElastoDyn, so that it runs faster

- Introduce tight coupling

- Introduce operating-point determination and linearization across the coupled aero-hydro-servo-elastic solution

- And much, much, more…

# Converting to FAST v8.09.x

We have created template input files for FAST v8.08.x, ElastoDyn v1.01.x, ServoDyn v1.01.x, and HydroDyn v2.02.x. These template files can be found in the Matlab Simulation Toolbox that is now included in the FAST archive:  Utilities\SimulationToolbox\ConvertFASTVersions\TemplateFiles.

See the “Matlab Conversion Scripts” section below for help in automatically converting input files to the latest version.

Also note that you can find example up-to-date input files in the FAST v8.09.00c-bjj archive’s CertTest folder. See the “Certification Tests” section of this document for descriptions.

## Summary of Changes to Inputs

This section summarizes changes to the FAST primary input file between major releases.

### Changes in FAST v8.09.00c-bjj

The primary input file of FAST v8.09.00a-bjj is the same as that of FAST v8.08.00c-bjj. However, the HydroDyn input file in this new version has one less line that the previous version.

### Changes in FAST v8.08.00c-bjj

The following list describes the differences in the primary input file of FAST v8.08.00c-bjj relative to FAST v8.03.02b-bjj.

- Many variables in the primary FAST input file have been renamed:

| FAST v8.03.x Name | FAST v8.08.x Name |
| --- | --- |
| ADFile | AeroFile |
| SrvDFile | ServoFile |
| HDFile | HydroFile |
| SDFile | SubFile |
| MAPFile | MooringFile |
| CompMAP | CompMooring |

- Most of the feature flags were changed to feature switches in the primary FAST input file. Instead of True/False inputs, CompAero, CompServo, CompHydro, CompSub, and CompMooring now require integer inputs.

- Inputs for coupling with modules IceFloe and IceDyn have been added: CompIce and IceFile.

- Several new inputs for future coupling of BeamDyn into FAST have been added. These inputs are CompElast, BDBldFile(1), BDBldFile(2), and BDBldFile(3).

- The modules ElastoDyn, ServoDyn, AeroDyn, HydroDyn, and SubDyn allow users to input the string “Default” for their respective time steps, which will then use the time step from the FAST primary input file.

### Changes in FAST v8.03.02b-bjj

The following list describes the differences in the FAST, ElastoDyn, and ServoDyn input files of FAST v8.03.02b-bjj relative to the input files of FAST v7.02.00d-bjj.

- The primary FAST input file has been converted to primary input files for FAST, ElastoDyn, and ServoDyn and some of the inputs have been reordered.

- The FAST Platform file has been eliminated, with some of the inputs now part of the ElastoDyn primary input file and some of the inputs now part of HydroDyn’s and MAP’s input files.

- All of the inputs formerly labeled “[CURRENTLY IGNORED]” have been removed.

- Switches for ADAMS preprocessing and linearization have been removed.

- Noise has been removed.

- PtfmLdMod has been converted to CompUsrPtfmLd.

- TwrLdMod has been converted to CompUserTwrLd.

- The tip-brake inputs have been removed.

- PtfmCM is now PtfmCMzt, with PtfmCMzt = -PtfmCM.

- Corresponding inputs PtfmCMxt and PtfmCMyt have been added.

- PtfmRef is now PtfmRefzt, with PtfmRefzt = -PtfmRef.

- TwrRBHt and TwrDraft have been replaced with TowerBsHt, with 
TowerBsHt = TwrRBHt – TwrDraft.

- The output decimation factor (DecFact) has been converted to DT_out (DT_out = DT*DecFact).

- The yaw and pitch maneuvers no longer specify end times for the maneuvers. Instead they specify a rate for the maneuver.

- The GBRevers variable has been removed; input GBRatio must now be specified as a negative number if GBRevers was previously set to True.

- ElastoDyn’s blade input properties table no longer specifies AeroCent. Instead, it specifies the location of the pitch axis, PitchAxis, which is calculated as PitchAxis = 0.5 – AeroCent by the MATLAB conversion script; the aerodynamic center will become part of AeroDyn in a future release.

- The OutList variables have been divided among the various FAST modules, and several outputs are no longer valid.

- The GH Bladed Interface is now a standard option in ServoDyn, without requiring a recompile.

- Tower drag loading has been added to AeroDyn v14.02.00c-mlb with a new corresponding flag in the AeroDyn input file.

- The glue code allows options for AbortErrLevel, number of corrections in the predictor-corrector algorithm, and extrapolation/interpolation order of module inputs to be used for time advancement.

- For the ElastoDyn coupling to HydroDyn or SubDyn, FAST also has two inputs controlling the implicit solve (via Jacobian computed with finite differences)—see DT_UJac and UJacSclFact described in the Variables Specified in the FAST Primary Input File Section above.

## Matlab Conversion Scripts

Because the changes to the input files are significant, we have created Matlab scripts to automatically convert FAST v7.x or FAST 8.03.02b-bjj input files to FAST v8.09.00a-bjj. The files you will need are included in the Simulation Toolbox, located in this directory of the FAST archive: Utilities\SimulationToolbox\ConvertFASTVersions.

We recommend that you add the Simulation Toolbox to your Matlab path so that you can access all of the routines defined in it. For example:

FASTSimulationToolbox = 'C:\Users\bjonkman\FAST\UtilityCodes\SimulationToolbox';

addpath( genpath( FASTSimulationToolbox ) );

An example showing how we converted the NREL CertTest input files for use with FAST v8.09.00a-bjj is included in the FAST archive: CertTest\ConvertFiles.m. You can use this script as a basis for helping to convert your own input files; however, we strongly recommend that you make copies of all your input files before running any scripts to convert them. Fortran and MATLAB read text input files in slightly different ways; so some things that may work in Fortran may not be read in the same way in MATLAB. We recommend that you carefully check the files after converting them.

### Converting from FAST v8.03.02b-bjj and v8.08.00c-bjj

If you have FAST v8.03.x input files that you wish to convert to FAST v8.09.00a-bjj, you can use the Matlab function, ConvertFAST8_3to8. This function will convert the primary FAST input file and the primary HydroDyn input file from FAST v8.03.x to FAST v8.08.x. You will need to provide the name of the FAST v8.03.x primary input file and the name of the directory where the new input files should be placed:

ConvertFAST8_3to8( inputfile, newDir );

Note that we do not automatically convert the SubDyn input files from FAST v8.03. These you must do by hand—or use the models provided in the FAST Certification Tests.

You can then call ConvertFAST8_8to9 to convert the FAST v8.08.x files for use with FAST v8.09.x.

ConvertFAST8_8to9( inputfile, newDir );

### Converting from FAST v7

To convert FAST v7 input files to FAST v8.09.00c-bjj, you can use the MATLAB function ConvertFAST7to8. This function will create a new primary input files for FAST, ServoDyn, and ElastoDyn. It also creates new blade and tower files for ElastoDyn. It does not automatically convert hydrodynamic- and mooring system-related inputs. This function does not create HydroDyn, SubDyn, MAP, or AeroDyn input files.

You will need to provide the ConvertFAST7to8 function with the name of the FAST v7.x primary input file and the directory where the new input files should be placed. The new directory should not be the directory where the old files are located!

ConvertFAST7to8(inputfile,newDir);

If your input file has pitch or yaw maneuvers, you may also provide the routine with the new rates (instead of the end times previously used). We have also provided a Matlab routine (CalculateYawAndPitchRates) that will calculate these rates, but you must provide the routine the name of the FAST output file that contains the previous results of the Pitch and/or Yaw channels.

[YawManRat, PitManRat] = CalculateYawAndPitchRates('Test09.fst', 'Test09.out');

ConvertFAST7to8(inputfile, newDir, YawManRat, PitManRat);

If YawManRat or PitManRat are zero, those inputs are ignored and values from the ServoDyn template file will be used instead.

If your input file was used with the custom interface to GH Bladed DLL controllers, you should also set the optional input parameter, usedBladedDLL, so that your input switches that previously called the DLL are now set to 5, the new switch for User-Defined Control from Bladed DLL.

ConvertFAST7to8(oldFSTName, newDir, YawManRat, PitManRat, usedBladedDLL)

# Compiling

If you want to compile the code, please read the file, “CompilingInstructions_FASTv8.pdf” located in the “Compiling” folder in the FAST archive for details.

Unlike FAST v7 distributions, all of the source code you need to compile the project is contained in the archive’s Source directory. The text files (.txt) in the source folders are input files for the FAST Registry. These files are used to generate the *_Types.f90 files for the component modules.

The compiling folder contains:

- A Microsoft Visual Studio project with all of the FAST source files and settings needed to compile in Release or Debug mode. This project places executables called “FAST_dev_Win32.exe” and “FAST_dev_debug_Win32.exe” in the FAST\bin folder. (renamed with “_dev” to prevent overwriting the executables created by NREL and distributed with FAST).

- A Windows® batch script that can be run from your Intel Fortran Command Prompt Window, with very little (if any) modification. This batch file creates executables named “FAST_iwin32.exe” and “FAST_iwin64.exe” in the local (i.e., FAST\Compiling) folder.

- A makefile for gfortran with (most of) the settings to create “FAST_gwin32.exe” and “FAST_gwin64.exe” in the local (i.e., FAST\Compiling) folder. To run this on Windows, you will need to install binaries of the LAPACK libraries. Please see “CompilingInstructions_FASTv8.pdf” for details. This makefile has been tested only on Windows. Also note that offshore models do not run with the gfortran executables (land-based models do).

All of these tools for compiling are set up to compile and link with the appropriate settings, though you may have to modify the makefile to find the LAPACK libraries. (Note that FAST uses several specialized compiling/linking options and that MAP is distributed as a dynamic-link library.) These tools also run the FAST Registry if necessary (e.g., changes to the Registry input files or missing *_Types.f90 files).

The Visual Studio project and Windows batch script were created using

Microsoft Visual Studio 2010 Shell
Intel® Visual Fortran Composer XE 2011, version 12.1.3.300
Intel Math Kernel Library (MKL) 10.3 Update 9

The MKL is used only for LAPACK routines.

# Running FAST v8.09.00a-bjj

FAST v8.09.00a-bjj must load the MAP library when the program starts. FAST_Win32.exe needs to load MAP_Win32.dll and FAST_x64.exe needs to load MAP_x64.dll. These dynamic link libraries (DLLs) must be on your Windows® path. The easiest way to do this is to make sure that the MAP DLLs are in the same directory as the FAST executables. We distribute the executables and DLLs in the \bin directory of the FAST archive, so this is already done for you. However, if you choose to move the files or if you compile using the Windows® batch script or the makefile for gfortran, you may have to modify your path environment variable or move some files.

To run FAST from a Windows command prompt, the syntax is:

<name of FAST executable with or without extension> <name of input file with extension>

To start, it easiest to open up your command window in the directory in which your FAST primary input file and FAST executable are stored. For example, if you have an input file named “Input.fst”, along with “FAST_Win32.exe”, stored in “C:\FileLocation”, you should type:

C:\>cd FileLocation

C:\FileLocation> FAST_Win32 Input.fst

The syntax is the same for different input files. Simply change “Input.fst” to whatever input file you want.

An installation guide is available that describes how to install FAST (and the other CAE tools) in such a way that they will run from a command window from any folder (without moving or copying the executable around to different folders). See: https://nwtc.nrel.gov/system/files/Setup_NWTC_Windows.pdf.

# FAST v8 Interface to Simulink

# FAST v8 has been implemented as a library that can be called from a Simulink S-Function block, using predefined inputs from Simulink. Simulink is a popular simulation tool for controls design that is distributed by The Mathworks, Inc. in conjunction with MATLAB

## Definition of the FAST v8 Interface to Simulink

The FAST v8 interface to Simulink is implemented as a level 2 S-Function. The interface is written in C, and it calls a DLL of FAST v8 routines, which is written in Fortran.

Matlab / Simulink
32-bit Windows 
64-bit Windows

Level 2 S Function
FAST_SFunc.mexw32
FAST_SFunc.mexw64
written in C

FAST Dynamic Library
FAST_Library_Win32.dll
FAST_Library_x64.dll
written in Fortran 2003

MAP Dynamic Library
MAP_Win32.dll
MAP_x64.dll
written in C

Figure 5: Libraries in the FAST - Simulink Interface

### S-Function Parameters

FAST_SFunc is designed to accept exactly three parameters from Simulink.

#### FAST_InputFileName

The first parameter is a string, which contains the name of the FAST v8 primary input file. In the sample models, this string is contained in a variable called FAST_InputFileName.

#### TMax

The second parameter is a double-precision real value called TMax. Because Simulink needs TMax to know the simulation length (the variable is often used for the simulation stop time), this value is passed to FAST_SFunc, even though it is also defined in the FAST input file. Any modules that use TMax inside FAST v8 (e.g., HydroDyn’s waves) will use the larger of the TMax value defined by Simulink or in the FAST v8 primary input file.

#### NumAdditionalInputs

NumAdditionalInputs is a value defined in the third parameter sent to the S-Function block. This value is set at initialization and cannot be changed during simulation. Currently, NumAdditionalInputs is 0 for most cases.

This third parameter can also be input as an array with no more than 11 entries. If this parameter is an array, the first element of the array is NumAdditionalInputs.

Currently, setting NumAdditionalInputs to 1, 2, or any number larger than 11 will probably result in a segmentation fault.

Setting NumAdditionalInputs = 3 causes the code to simulate a CW lidar, whose measurement is set in an output channel called “WindMeas1”. When NumAdditionalInputs = 3, the code expects this third parameter to contain an array of initialization data for the lidar module:

[3, UseLidar, LidRadialV]

UseLidar should be 0 (single point measurement; no lidar) or 1 (CW lidar). Future version will allow more flexibility.

LidRadialV should be 0 (returns estimate of velocity in x direction) or 1 (returns radial velocity). Other values for LidRadialV will be treated as 0.

### S-Function Inputs

The inputs to the FAST S-Function are values in an array of size 7 + NumAdditionalInputs. (See section “S-Function Parameters”)

The values in the input array are as follows:

- Generator torque

- Electrical power

- Commanded yaw position

- Commanded yaw rate

- Commanded pitch for blade 1

- Commanded pitch for blade 2

- Commanded pitch for blade 3 (unused on 2-bladed turbines, but required as input to DLL)

- Lidar focus position (w.r.t lidar) in X direction (required for continuous wave lidar)

- Lidar focus position (w.r.t lidar) in Y direction (required for continuous wave lidar)

- Lidar focus position (w.r.t lidar) in Z direction (required for continuous wave lidar)

Note that these inputs are passed from Simulink to ServoDyn (and InflowWind’s lidar module). However, these inputs are used only if the appropriate switches in the ServoDyn input file are selected. The PCMode, VSContrl, and YCMode parameters in the ServoDyn input file must be set to “4” to allow the inputs from Simulink to be used for pitch control, variable-speed control, and/or nacelle yaw control, respectively.

### S-Function Outputs

The outputs from FAST to Simulink are the values that are written to the FAST output file(s). These are the channels defined in module OutList variables; these channel names and units are also written in the FAST summary file. At the FAST_SFunc block initialization, FAST_SFunc writes a cell array called “ÓutList” containing the names of these output channels to the Matlab base workspace.

### S-Function States

The FAST v8 S-Function does not register any states with Simulink. This effectively makes the block a discrete system which is solved inside the S-Function block. Whereas the FAST v7 Simulink models frequently required a time-delay block to allow the system to solve, FAST v8 Simulink models should not need to implement a time-delay block. Currently, though, there appears to be a time delay of the inputs from Simulink inside the FAST v8 block. (Need to schedule a meeting with Mathworks [Tony Lennon] to discuss if this is the best approach.)

## Converting FAST v7 Simulink Models to FAST v8

### Convert FAST input file(s)

### Any FAST 7 input files must be converted to files for FAST v8 prior to using the FAST v8 Interface to Simulink. See the section “Converting to FAST v8.09.x” for help with this conversion.

### Edit Simulink model

- Remove state integrations from the FAST Nonlinear Wind Turbine block

- Remove time delay (if it exists)

- Change pitch controller to input 3 values instead of NumBl values

- There are no continuous states, so the solver may be changed to FixedStepDiscrete.

- If the Simulink model previously used states (q, qdot) from FAST v7, choose the appropriate state outputs from ElastoDyn instead (see OutListParameters.xlsx).

- Replace any previously used output channels that don’t exist in FAST v8 (see “FAST v7 control outputs that don’t exist in FAST v8”).

- Add appropriate inputs to the FAST_SFunc block (see “S-Function Parameters”).

### FAST v7 control outputs that don’t exist in FAST v8

Some output channels that existed in FAST v7 do not exist in FAST v8. Some substitutions for variables commonly used in control include:

- HorWindV = WindVxi2  +WindVyi2

- HorWindDir = 180°πatan2 WindVyi,WindVxi

- VerWindDir = 180°πatan2 WindVzi, atan2 WindVyi,WindVxi

- NacYawErr = HorWindDir-YawPzn-YawBrRDzt- PtfmRDzi

To use these equations, replace variable names on the right-hand-side of the above equations using ustrmatch'VariableName',OutList.

### Initialization

The Simulink interface for FAST v8 does not read any variables defined in the Matlab workspace, which means the Read_FAST_Input.m file does not exist in v8. All values required for the DLL to run are passed directly through S-Function parameters. (See section “S-Function Parameters”).

After FAST v8 is initialized, it places two variables, OutList, and DT in the Matlab workspace. OutList is a cell array of channel names corresponding to the FAST output channels, and DT is a double-precision real value that is required for the Simulink solver.

Depending on the model, you may have to set DT and OutList prior to running your Simulink model. The two sample models included in the FAST archive do not require this step, but more complicated models may; this is because the FAST_SFunc block may not be initialized before Simulink checks that all other blocks are valid. OutList and DT will be overwritten before FAST_SFunc calculates any output, so they do not necessarily have to be correct when initialized; they just have to exist.

### Output Files

Output files from FAST v8 for Simulink are named <RootName>.SFunc.<ext>. This is in contrast to files from FAST v7, which were named <RootName>_SFunc.<ext>. Note that the extensions in FAST v8 have changed from what was used in FAST v7 (see section “v8.08.00c-bjj” under “Major changes in FAST”).

## Running FAST in Simulink

To run the FAST S-Function from Simulink, Matlab must be able to find the appropriate DLLs. This includes the FAST_SFunc.mex*, FAST_Library_*.dll, and MAP_*.dll files. All of these files are contained in the FAST archive’s bin directory, so the easiest way to do this is to add the <FAST8>\bin directory to the Matlab path.

### Sample Simulink Models for FAST v8

Two sample models for running FAST v8 with Simulink are provided in the FAST archive (see the <FAST8>/Simulink/Samples folder). These examples are intended to help the user understand how to use the FAST_SFunc block. It assumed that the user is already somewhat familiar with the Simulink environment.

#### OpenLoop

The OpenLoop sample model contains the FAST S-Function block and constant open loop control input blocks.

The Run_OpenLoop.m script in the <FAST8>/Simulink/Samples folder allows the user to run all of the FAST Certification Tests from Simulink using the OpenLoop model without using any of the control inputs from Simulink.

Figure 6: OpenLoop.mdl Sample Model for Simulink

Figure 7: FAST Nonlinear Wind Turbine Block in Simulink

#### Test01_SIG

The Test01_SIG.mdl file contains the FAST S-Function block and the simple induction generator model for FAST certification test #01 implemented within Simulink. To run this model, change VSControl to “4” in the <FAST8>/CertTest/AWT27/Test01_ServoDyn.dat file, then run the Run_Test01_SIG.m file in the Simulink/Samples folder.

Figure 8: Test01_SIG.mdl Sample Model for Simulink

## Error Messages

## If your Simulink model fails to run, please make note of any error/informational windows that open. Also make sure to look at any text written to the Matlab Command Window, which is where all messages from the FAST_Library_*.dll file will be written.

## Compiling FAST for Simulink

## The S-Function (mex) file contained in the

### FAST_Library

The FAST archive contains a sample Visual Studio Intel Fortran project (2010) that is set up to compile a dll called FAST_Library_{Win32 | x64}.dll and place it in the <FAST8>/bin folder. This project is located in the <FAST8>/Simulink/VisualStudio folder.

### Compiling the FAST library is very similar to compiling the stand-alone version of FAST v8, which is described in the “Compiling” section of this document. The remainder of this section describes some things that are different

### Two files need to be replaced to compile FAST as a dll:

### FAST_Library.f90 should be used in place of FAST_Prog.f90

### SysMatlab.f90 should be used in place of SysIVF.f90. (Note that this assumes we’re using the Intel Visual Fortran compiler. SysMatlab.f90 may need slight modification if using gfortran.)

### A few compiling commands are different from the stand-alone application:

### The project must create a dynamic library (DLL or shared object) instead of a console application.

### The COMPILE_SIMULINK preprocessor directive must be used.

### The FAST library should be linked with libmex.lib. SysMatlab.f90 is designed to call Matlab’s mexPrintf function to print in the Matlab Command Window. This function is part of the libmex.lib library, which is found in the %matlabroot%/extern/lib/{architecture}/{compiler}/ folder. For the 64-bit version of Matlab R2012, using Visual Studio, the file is located here:
     "C:\Program Files\MATLAB\R2012a\extern\lib\win64\microsoft\libmex.lib"

If you wish to compile the library without linking with libmex.lib, use the preprocessor directive CONSOLE_FILE. This directive will write to a text file called CONSOLE.TXT instead of to the Matlab Command Window

### Figure 9: Visual Studio window showing additional dependencies for the linker (MAP and libmex)

### Figure 10: Preprocessor directives for FAST_Library

### FAST_SFunc

### To compile the FAST_SFunc S-Function, you must have a C compiler supported by the version of Matlab you are using. The compilation is done in Matlab via the “mex” command. If you haven’t already done so, run “mex –setup” from the Matlab Command Window to select your compiler.

### For 32-bit versions of Matlab, the command is

mex -L../../bin -lFAST_Library_Win32 -outdir ../../bin LINKFLAGS='$LINKFLAGS /STACK:999999999 /LARGEADDRESSAWARE' FAST_SFunc.c

For 64-bit versions of Matlab it is

mex -v -L../../bin -lFAST_Library_x64 -outdir ../../bin FAST_SFunc.c

These commands are specified in the Simulink/Source/create_FAST_SFunc.m file. The script assumes it is being run from the <FAST8>/Simulink/Source/ directory.

These commands compile FAST_SFunc.c (which includes FAST_Library.h) and link it with the FAST_Library_*.lib file for the appropriate addressing scheme (32- or 64-bits). If you are using a 32-bit Windows® version of Matlab , this will produce ../../bin/FAST_SFunc.mexw32. On 64-bit Windows versions of Matlab, it will produce ../../bin/FAST_SFunc.mexw64.

Because FAST_SFunc.c passes the input/output arrays directly to the FAST_Library*.dll file, FAST_SFunc should not need to be modified (or recompiled) very often. For example, if you wanted to add an additional FAST_SFunc input from Simulink, you would change the third S-Function parameter in the S-Function block in Simulink and then modify FAST_Library.f90 to handle the new input value, without touching FAST_SFunc.c FAST_Library would need to be recompiled, but FAST_SFunc would not.

# Feedback

If you have questions or wish to provide feedback, please use our forums: https://wind.nrel.gov/forum/wind/

# Appendix: Example FAST v8.09.* Input File

------- FAST v8.09.* INPUT FILE ------------------------------------------------

NREL 5.0 MW Baseline Wind Turbine with OC4 Jacket Configuration, for use in offshore analysis

---------------------- SIMULATION CONTROL --------------------------------------

False      Echo           - Echo input data to <RootName>.ech (flag)

"FATAL"    AbortLevel     - Error level when simulation should abort (string) {"WARNING", "SEVERE", "FATAL"}

60  TMax           - Total run time (s)

0.01  DT             - Recommended module time step (s)

2  InterpOrder    - Interpolation order for input/output time history (-) {1=linear, 2=quadratic}

1  NumCrctn       - Number of correction iterations (-) {0=explicit calculation, i.e., no corrections}

99999  DT_UJac        - Time between calls to get Jacobians (s)

1E+06  UJacSclFact    - Scaling factor used in Jacobians (-)

---------------------- FEATURE SWITCHES AND FLAGS ------------------------------

1  CompElast      - Compute structural dynamics (switch) {1=ElastoDyn; 2=ElastoDyn+BeamDyn blades}

1  CompAero       - Compute aerodynamic loads (switch) {0=None; 1=AeroDyn}

1  CompServo      - Compute control and electrical-drive dynamics (switch) {0=None; 1=ServoDyn}

1  CompHydro      - Compute hydrodynamic loads (switch) {0=None; 1=HydroDyn}

1  CompSub        - Compute sub-structural dynamics (switch) {0=None; 1=SubDyn}

0  CompMooring    - Compute mooring system (switch) {0=None; 1=MAP; 2=FEAMooring}

0  CompIce        - Compute ice loads (switch) {0=None; 1=IceFloe; 2=IceDyn}

False      CompUserPtfmLd - Compute additional platform loading (flag) {false: none; true: user-defined}

False      CompUserTwrLd  - Compute additional tower loading (flag) {false: none; true: user-defined}

---------------------- INPUT FILES ---------------------------------------------

"OC4Jacket_ElastoDyn.dat" EDFile       - Name of file containing ElastoDyn input parameters (quoted string)

"unused"                  BDBldFile(1) - Name of file containing BeamDyn blade 1 inputs (quoted string)

"unused"                  BDBldFile(2) - Name of file containing BeamDyn blade 2 inputs (quoted string)

"unused"                  BDBldFile(3) - Name of file containing BeamDyn blade 3 inputs (quoted string)

"OC4Jacket_AeroDyn.dat"   AeroFile     - Name of file containing aerodynamic input parameters (quoted string)

"OC4Jacket_ServoDyn.dat"  ServoFile    - Name of file with control/electric-drive inputs (quoted string)

"OC4Jacket_HydroDyn.dat"  HydroFile    - Name of file containing hydrodynamic inputs (quoted string)

"OC4Jacket_SubDyn.dat"    SubFile      - Name of file containing sub-structural inputs (quoted string)

"unused"                  MooringFile  - Name of file containing mooring system inputs (quoted string)

"unused"                  IceFile      - Name of file containing ice input parameters (quoted string)

---------------------- OUTPUT --------------------------------------------------

True       SumPrint        - Print summary data to "<RootName>.sum" (flag)

1  SttsTime        - Amount of time between screen status messages (s)

0.05  DT_Out          - Time step for tabular output (s)

0  TStart          - Time to begin tabular output (s)

2  OutFileFmt      - Format for tabular (time-marching) output file (switch) {1:out, 2:outb, 3:both}

True       TabDelim        - Use tab delimiters in text tabular output file? (flag)

"ES10.3E2" OutFmt          - Format used for text tabular output, excluding the time channel. (quoted string)

Figure 11: Example FAST v8.09.00a-bjj Input File

This option is a custom feature in FAST v7, requiring a separate executable. In FAST v8, it is part of the standard distribution.

These steps must be integer multiples of the structural time step.

These steps must be integer divisors of the glue-code time step. Future versions will allow integer multiples of the glue-code time step as well.

FAST v7 is limited to one correction step and this correction step only applies to some modules.

The FAST v8.09 source code can be compiled using gfortran, however the offshore cases do not run with this compiled executable. We are working to find the problem and fix it.

The FAST Registry reads input files from each module to auto-generate the *_Types.f90 files required for the FAST framework.

IceDyn and FEAMooring have been added to FAST v8.09.00a-bjj, but they are not complete and have not been tested well.

If you use the “NEWTOWER” model in AeroDyn, you will have to add one additional line (the AeroDyn input file for Test18 in the CertTest folder contains this line); if you do not use the “NEWTOWER” model, the AeroDyn input file has not changed.